Lab 11: Single-Cycle Processor

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

Objective

In this lab, you will extend the single-cycle RISC-V processor from the textbook and lecture to support two additional instructions: lui and xor.

1. Multicycle RISC-V Processor

Figure 1 shows the complete single-cycle processor from the textbook. Figure 2 shows the Control Unit, and Figure 3 shows the ALU. Tables 1 and 2 are the Main Decoder and ALU Decoder truth tables. Table 3 lists the ImmSrc encoding. Figure 4 shows the test program for the RISC-V single-cycle processor in the textbook.

What to Turn In

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- A marked up version of Figure 1 (single-cycle processor) showing the needed modifications (lui and xor)

- A marked up version of Figures 2 and 3 (if modified) showing the needed modifications for the additional instructions (lui and xor)

- Amended Main Decoder, ALU Decoder, and ImmSrc truth tables to support lui and xor

- Amended SystemVerilog code to add support for lui and xor

- Modified test program that uses lui and xor

- Simulation waveforms (in the order listed above: clk, reset, PC, Instr, SrcA, SrcB, ALUResult, DataAdr, WriteData, and MemWrite – all displayed in hexadecimal for ease of reading). Does your system pass your testbench? Circle or highlight the waves showing that the correct value is written to the correct address, and make sure it is legible.

Please indicate any bugs you found in this lab manual, or any suggestions you would have to improve the lab.

Figure 1: RISC-V single-cycle processor

Figure 2: RISC-V single-cycle processor control unit

Figure 3: ALU

Table 1. Main Decoder Truth Table

| Instruction | Opcode | RegWrite | ImmSrc | ALUSrcA | ALUSrcB | MemWrite | ResultSrc | Branch | ALUOp | Jump |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lw | 0000011 | 1 | 00 | 0 | 1 | 0 | 01 | 0 | 00 | 0 |
| sw | 0100011 | 0 | 01 | 0 | 1 | 1 | xx | 0 | 00 | 0 |
| R-type | 0110011 | 1 | xx | 0 | 0 | 0 | 00 | 0 | 10 | 0 |
| beq | 1100011 | 0 | 10 | 0 | 0 | 0 | xx | 1 | 01 | 0 |
| I-type ALU | 0010011 | 1 | 00 | 0 | 0 | 0 | 00 | 0 | 10 | 0 |
| jal | 1101111 | 1 | 11 | x | x | 0 | 10 | 0 | xx | 1 |

Table 2. ALU Decoder Truth Table

| ALUOp1:0 | funct32:0 | {op5, funct75} | ALUControl2:0 | Operation |
| --- | --- | --- | --- | --- |
| 00 | x | x | 000 | Add |
| 01 | x | x | 001 | Subtract |
| 10 | 000 | 00, 01, 10 | 000 | Add |
|  | 000 | 11 | 001 | Subtract |
|  | 010 | x | 101 | SLT |
|  | 110 | x | 011 | OR |
|  | 111 | x | 010 | AND |

Table 3.  ImmSrc encoding

| ImmSrc | ImmExt | Type | Description |
| --- | --- | --- | --- |
| 00 | {{20{Instr[31]}}, Instr[31:20]} | I | 12-bit signed immediate |
| 01 | {{20{Instr[31]}}, Instr[31:25], Instr[11:7]} | S | 12-bit signed immediate |
| 10 | {{20{Instr[31]}}, Instr[7], Instr[30:25], Instr[11:8], 1’b0} | B | 13-bit signed immediate |
| 11 | {{12{Instr[31]}}, Instr[19:12], Instr[20], Instr[30:21], 1’b0} | J | 21-bit signed immediate |

#       RISC-V Assembly         Description               Address   Machine

main:   addi x2, x0, 5          # x2 = 5                  0         00500113

addi x3, x0, 12         # x3 = 12                 4         00C00193

addi x7, x3, -9         # x7 = (12 - 9) = 3       8         FF718393

or   x4, x7, x2         # x4 = (3 OR 5) = 7       C         0023E233

and  x5, x3, x4         # x5 = (12 AND 7) = 4     10        0041F2B3

add  x5, x5, x4         # x5 = (4 + 7) = 11       14        004282B3

beq  x5, x7, end        # shouldn't be taken      18        02728863

slt  x4, x3, x4         # x4 = (12 < 7) = 0       1C        0041A233

beq  x4, x0, around     # should be taken         20        00020463

addi x5, x0, 0          # shouldn't happen        24        00000293

around: slt  x4, x7, x2         # x4 = (3 < 5)  = 1       28        0023A233

add  x7, x4, x5         # x7 = (1 + 11) = 12      2C        005203B3

sub  x7, x7, x2         # x7 = (12 - 5) = 7       30        402383B3

sw   x7, 84(x3)         # [96] = 7                34        0471AA23

lw   x2, 96(x0)         # x2 = [96] = 7           38        06002103

add  x9, x2, x5         # x9 = (7 + 11) = 18      3C        005104B3

jal  x3, end            # jump to end, x3 = 0x44  40        008001EF

addi x2, x0, 1          # shouldn't happen        44        00100113

end:    add  x2, x2, x9         # x2 = (7 + 18)  = 25     48        00910133

sw   x2, 0x20(x3)       # mem[100] = 25           4C        0221A023

done:   beq  x2, x2, done       # infinite loop           50        00210063

Figure 4. RISC-V test program