Lab 2: FPGA Tools & Combinational Logic Design SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- [2] Boolean equations for your ALU Decoder

ALUControl[2] = ALUOp[1] & ~funct3[2] & funct3[1]

ALUControl[1] = ALUOp[1] &  funct3[2] & funct3[1]

ALUControl[0] = ~ALUOp[1] & ALUOp[0] | ALUOp[1] &

(~funct3[2] & ~funct3[1] & ~funct3[0] & opb5 & funct7b5 |

funct3[1]&~funct3[0]);

- [1] Gate-level schematic of your ALU Decoder

- [2] Structural Verilog code for your ALU Decoder

module aludec(input  logic       opb5,

input  logic [2:0] funct3,

input  logic       funct7b5,

input  logic [1:0] ALUOp,

output logic [2:0] ALUControl);

logic [3:0] funct3b; // Inverse of funct3 - Verilog won't implicitly declare arrays

logic n1, n2, n3, n4, n5; // Best practice to explicitly declare all internal signals

logic ALUOpb1b;

not not1(ALUOpb1b, ALUOp[1]);

not not2(funct3b[2], funct3[2]);

not not3(funct3b[1], funct3[1]);

not not4(funct3b[0], funct3[0]);

and a1(ALUControl[2], ALUOp[1], funct3b[2], funct3[1]);

and a2(ALUControl[1], ALUOp[1], funct3[2], funct3[1]);

and a3(n1, ALUOpb1b, ALUOp[0]);

and a4(n2, funct3b[2], funct3b[1], funct3b[0], opb5, funct7b5);

and a5(n3, funct3[1], funct3b[0]);

or  o1(n4, n2, n3);

and a6(n5, ALUOp[1], n4);

or  o2(ALUControl[0], n1, n5);

endmodule

Graders: please give full credit for using inline NOTs (e.g. ~funct3[2] in place of fb[2]), although this is technically mixing structural and behavioral Verilog.

- [1] RTL Viewer schematics of your synthesized ALU Decoder

- [1] Self-checking test bench for your ALU Decoder with a test vector file

module testbench_aludec();

logic        clk, reset;

logic        opb5;

logic [2:0]  funct3;

logic        funct7b5;

logic [1:0]  ALUOp;

logic [2:0]  ALUControl, ALUControlexpected;

logic [31:0] vectornum, errors;

logic [9:0]  testvectors[10000:0];

// instantiate device under test

aludec dut(opb5, funct3, funct7b5, ALUOp, ALUControl);

// generate clock

always

begin

clk = 1; #5; clk = 0; #5;

end

// at start of test, load vectors

// and pulse reset

initial

begin

$readmemb("aludec.tv", testvectors);

vectornum = 0; errors = 0;

reset = 1; #22; reset = 0;

end

// apply test vectors on rising edge of clk

always @(posedge clk)

begin

#1; {ALUOp, funct3, opb5, funct7b5, ALUControlexpected} = testvectors[vectornum];

end

// check results on falling edge of clk

always @(negedge clk)

if (~reset) begin // skip during reset

if (ALUControl !== ALUControlexpected) begin  // check result

$display("Error: inputs = %b, %b, %b, %b", ALUOp, funct3, opb5, funct7b5);

$display("  outputs = %b (%b expected)",ALUControl, ALUControlexpected);

errors = errors + 1;

end

vectornum = vectornum + 1;

if (testvectors[vectornum] === 10'bx) begin

$display("%d tests completed with %d errors",

vectornum, errors);

$stop;

end

end

endmodule

Test vectors

// ALUOp_funct3_opb5_funct7b5_ALUControlexpected

00_000_00_000

01_011_11_001

10_000_00_000

10_000_01_000

10_000_10_000

10_000_11_001

10_010_01_101

10_110_00_011

10_111_11_010

- [2] Simulation waveforms showing the ALU Decoder simulation. Display the signals in binary in this order (top to bottom): ALUOp, funct3, opb5, funct7b5, ALUControl, ALUControlExpected. Did it work correctly? YES

- [1] Did the ALU Decoder function correctly on the DE0-CV Board? YES