Lab 3: Finite State Machine Design SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- [2] State transition diagram

- [2] State encoding and Boolean equations for next state and output

Use one-hot encoding

NS[0] = S[0] & ~left & ~right | S[3] | S[6];

NS[1] = S[0] & left;

NS[2] = S[1];

NS[3] = S[2];

NS[4] = S[0] & right;

NS[5] = S[4];

NS[6] = S[5];

LA = S[1] | S[2] | S[3];

LB = S[2] | S[3];

LC = S[3];

RA = S[4] | S[5] | S[6];

RB = S[5] | S[6];

RC = S[6];

- [2] Sketch of circuit

- [2] Structural SystemVerilog code.

Note: question specifically asked for structural SystemVerilog.  If the solution uses assign statements, Don’t deduct points here but indicate that they should have used structural SystemVerilog.

module lab3_dh(input logic clk,

input logic reset,

input logic left, right,

output logic la, lb, lc, ra, rb, rc);

logic [6:0] state, nextstate;

logic       leftb, rightb, stay0;

// state register

always @(posedge clk or posedge reset)

if (reset) state <= 7'b0000001;

else       state <= nextstate;

// nextstate logic

not nl(leftb, left);

not nr(rightb, right);

and as0(stay0, state[0], leftb, rightb);

or  os0(nextstate[0], state[3], state[6], stay0);

and as1(nextstate[1], state[0], left);

buf bf2(nextstate[2], state[1]);

buf bf3(nextstate[3], state[2]);

and as4(nextstate[4], state[0], right);

buf bf5(nextstate[5], state[4]);

buf bf6(nextstate[6], state[5]);

// output logic

or  ola(la, state[1], state[2], state[3]);

or  olb(lb, state[2], state[3]);

buf blc(lc, state[3]);

or  ora(ra, state[4], state[5], state[6]);

or  orb(rb, state[5], state[6]);

buf brc(rc, state[6]);

endmodule

- [2] Simulation waveforms showing the FSM inputs and outputs.  Did it pass the self-checking testbench?  YES

- [1] RTL Viewer schematics.  Do they match your expectations?  YES

- [2] Did the tail lights function correctly on the DE0-CV Board? YES