Lab 4: 7-Segment Display SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- Completed Table 1.

Table 1. Truth table for 7-segment display decoder

| Hexadecimal Digit | Inputs |  |  |  | Outputs |  |  |  |  |  |  | (in hex) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D3 | D2 | D1 | D0 | Sg | Sf | Se | Sd | Sc | Sb | Sa |  |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 40 |
| 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 79 |
| 2 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 24 |
| 3 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 30 |
| 4 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 19 |
| 5 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 12 |
| 6 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 02 |
| 7 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 78 |
| 8 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 00 |
| 9 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 18 |
| A | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 08 |
| B | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 03 |
| C | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 27 |
| D | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 21 |
| E | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 06 |
| F | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0E |

3.	SystemVerilog modules: sevenseg.sv and sevenseg_wrapper.sv.

sevenseg.sv:

module sevenseg(input  logic [3:0] data,

output logic [6:0] segments);

always_comb

case (data)         //Sg - Sa

4'h0: segments = 7'b1000000;

4'h1: segments = 7'b1111001;

4'h2: segments = 7'b0100100;

4'h3: segments = 7'b0110000;

4'h4: segments = 7'b0011001;

4'h5: segments = 7'b0010010;

4'h6: segments = 7'b0000010;

4'h7: segments = 7'b1111000;

4'h8: segments = 7'b0000000;

4'h9: segments = 7'b0011000;

4'hA: segments = 7'b0001000;

4'hB: segments = 7'b0000011;

4'hC: segments = 7'b0100111;

4'hD: segments = 7'b0100001;

4'hE: segments = 7'b0000110;

4'hF: segments = 7'b0001110;

endcase

endmodule

sevenseg_wrapper.sv:

module sevenseg_wrapper(input  logic [3:0] SW,

output logic [6:0] HEX0);

sevenseg sseg1(SW, HEX0);

endmodule

- Testbench module and test vector files (testbench_sevenseg.sv and sevenseg.txt).

testbench_sevenseg.sv:

module testbench_sevenseg();

logic        clk, reset;

logic [6:0]  segments, segments_expected;

logic [3:0]  data;

logic [31:0] vectornum, errors;

logic [10:0] testvectors[10000:0];

// instantiate device under test

sevenseg dut(data, segments);

// generate clock

always

begin

clk = 1; #5; clk = 0; #5;

end

// at start of test, load vectors

// and pulse reset

initial

begin

$readmemh("sevenseg.txt", testvectors);

vectornum = 0; errors = 0;

reset = 1; #27; reset = 0;

end

// apply test vectors on rising edge of clk

always @(posedge clk)

begin

#1; {segments_expected, data} = testvectors[vectornum];

end

// check results on falling edge of clk

always @(negedge clk)

if (~reset) begin // skip during reset

if (segments !== segments_expected) begin  // check result

$display("Error: inputs = %h", data);

$display("  outputs = %h (%h expected)", segments,

segments_expected);

errors = errors + 1;

end

vectornum = vectornum + 1;

if (testvectors[vectornum] === 11'bx) begin

$display("%d tests completed with %d errors",

vectornum, errors);

$stop;

end

end

endmodule

sevenseg.txt:

// segments_data

40_0

79_1

24_2

30_3

19_4

12_5

02_6

78_7

00_8

18_9

08_A

03_B

27_C

21_D

06_E

0E_F

5.	Simulation waveforms showing the inputs (data) and outputs (segments) in that order, from top to bottom, and in hexadecimal.  Do they pass your testbench and match expectations? YES.

6.	RTL Viewer schematics of your sevenseg_wrapper.sv and sevenseg.sv modules (two schematics). Do they match your expectation? YES.

sevenseg_wrapper.sv:

sevenseg.sv:

Please indicate any bugs you found in this lab manual, or any suggestions you would have to improve the lab.