Lab 5: Finite State Machine Design SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- [1] State transition diagrams for the room and sword FSMs.

3.      [3] Behavioral SystemVerilog code for the system.

module lab4_dh(input  logic clk, reset,

input  logic n, s, e, w,

output logic win, die);

logic hassword, getsword;

roomfsm room(clk, reset, n, s, e, w, hassword, getsword, win, die);

swordfsm sword(clk, reset, getsword, hassword);

endmodule

module roomfsm(input  logic clk, reset,

input  logic n, s, e, w,

input  logic hassword,

output logic getsword,

output logic win, die);

typedef enum logic [2:0] {CC, TT, RR, SS, DD, GG, VV} roomtype;

roomtype state, nextstate;

// state register

always_ff @(posedge clk, posedge reset)

if (reset) state <= CC;

else       state <= nextstate;

// next state logic; takes advantage of assumption that one

// legal input is asserted on each cycle

always_comb

case(state)

CC:        nextstate <= TT;

TT: if (w) nextstate <= CC;

else   nextstate <= RR;

RR: if (w) nextstate <= SS;

else   nextstate <= DD;

SS:        nextstate <= RR;

DD: if (~hassword) nextstate <= GG;

else if (w) nextstate <= RR;

else   nextstate <= VV;

GG:        nextstate <= GG;

VV:        nextstate <= VV;

default:   nextstate <= CC;

endcase

// output logic

assign getsword = (state == SS);

assign die = (state == GG);

assign win = (state == VV);

endmodule

module swordfsm(input  logic clk, reset,

input  logic getsword,

output logic hassword);

logic state, nextstate;

// state register

always @(posedge clk, posedge reset)

if (reset) state <= 0;

else       state <= nextstate;

assign nextstate = state | getsword;

assign hassword = state;

endmodule

4. 	[2] A single testbench and test vectors illustrating both the win and die cases.

module testbench();

logic        clk, reset;

logic        n, s, e, w, win, die, winexpected, dieexpected;

logic [31:0] vectornum, errors;

logic [5:0]  testvectors[10000:0];

// instantiate device under test

lab4_dh dut(clk, reset, n, s, e, w, win, die);

// generate clock

always

begin

clk=1; #5; clk=0; #5;

end

// at start of test, load vectors

// and pulse reset

initial

begin

$readmemb("lab4.tv", testvectors);

vectornum = 0; errors = 0; reset = 1; #22; reset = 0; #70; reset = 1; #10; reset = 0;

end

// apply test vectors on rising edge of clk

always @(posedge clk)

begin

#1; {n, s, e, w, winexpected, dieexpected} = testvectors[vectornum];

end

// check results on falling edge of clk

always @(negedge clk)

if (~reset) begin    // skip during reset

if (win !== winexpected || die !== dieexpected) begin // check result

$display("Error: inputs = %b", {n, s, e, w});

$display(" state = %b", dut.room.state);

$display(" outputs = %b %b (%b %b expected)",

win, die, winexpected, dieexpected);

errors = errors + 1;

end

vectornum = vectornum + 1;

if (testvectors[vectornum] === 6'bx) begin

$display("%d tests completed with %d errors", vectornum, errors);

$stop;

end

end

endmodule

5.	[1] Simulation waveforms including the inputs, outputs, and current room.  Do they pass your testbench and match expectations?  YES

6.	[2] RTL Viewer schematics (including each FSM).  Do they match your expectation? YES

7.  	[1 point, 2 for something very elaborate] EXTRA CREDIT: It is a little known fact that the Twisty Tunnel is located beneath Hoch and that by heading north one can reach the dormitories.  Extend your adventure game with more interesting rooms or objects.  There will be a prize for the most interesting working enhancement!

If you have suggestions for further improvements of this lab, you’re welcome to include them at the end of your lab.

Spjut’s alternate solution

Behavioral Verlog for system

module room(input logic clk, reset,

input logic [3:0] direction, // mapping N--0001[0], E--0010[1], S--0100[2], W--1000[3]

input logic havesword,

output logic getsword, youwon, youredead);

typedef enum logic [3:0] {CC, TT, RR, SSS, DD, GG, VV} statetype; // left room for expansion to more states

statetype state, nextstate;

// state register

always_ff @(posedge clk, posedge reset)

if (reset) state <= CC;

else       state <= nextstate;

// next state logic

always_comb

case (state)

CC: if (direction[1]) nextstate = TT;

else nextstate = CC; // the else is not strictly needed according to lab

TT: if (direction[2]) nextstate = RR;

else if (direction[3]) nextstate = CC;

else nextstate = TT; // the else is not strictly needed according to lab

RR: if (direction[0]) nextstate = TT;

else if (direction[1]) nextstate = DD;

else if (direction[3]) nextstate = SSS;

else nextstate = RR; // the else is not strictly needed according to lab

SSS: if (direction[1]) nextstate = RR;

else nextstate = SSS; // the else is not strictly needed according to lab

DD: if (~havesword) nextstate = GG;

else if (direction[1]) nextstate = VV;

else if (direction[3]) nextstate = RR;

else nextstate = DD; // the else is not strictly needed according to lab

GG: nextstate = GG;

VV: nextstate = VV;

default: nextstate = CC;

endcase

// output logic

assign youredead = (state == GG);

assign youwon = (state == VV);

assign getsword = (state == SSS);

endmodule

module sword(input logic clk, reset,

input logic getsword,        // inss is in Secret Sword Stash

output logic havesword);

logic state, nextstate;

// state register

always_ff @(posedge clk, posedge reset)

if (reset) state <= 0;        // 0 is don't have sword state

else       state <= nextstate;

// next state logic

always_comb

if (getsword) nextstate = 1;

else nextstate = state;

// output logic

assign havesword = state;

endmodule

module adventure(input logic clk, reset,

input logic [3:0] direction, // mapping N--0001[0], E--0010[1], S--0100[2], W--1000[3]

output logic youwon, youredead);

logic getsword, havesword;

sword s1(clk, reset, getsword, havesword);

room r1(clk, reset, direction, havesword, getsword, youwon, youredead);

endmodule

Behavioral Verilog for testbench

module testbench();

logic clk, reset;

logic [3:0] direction;

logic youwon, youredead, youwonexpected, youredeadexpected;

logic [31:0] vectornum, errors;

logic [5:0] testvectors[10000:0];

// instantiate device under test

adventure dut(clk, reset, direction, youwon, youredead);

// generate clock

always

begin

clk=1; #5; clk=0; #5;

end

// at start of test, load vectors

// and pulse reset

initial

begin

$readmemb("adventure.tv", testvectors);

vectornum = 0; errors = 0; reset = 1; #22; reset = 0; #100; reset = 1; #10; reset = 0; //fix reset

end

// apply test vectors on rising edge of clk

always @(posedge clk)

begin

#1; {direction, youwonexpected, youredeadexpected} = testvectors[vectornum];

end

// check results on falling edge of clk

always @(negedge clk)

if (~reset) begin // skip during reset

$display("To travel in %b direction. Currently in room %s. Sword=%b Dead=%b, Won=%b",

direction, dut.r1.state.name, dut.r1.havesword, youredead, youwon);

if (youwon !== youwonexpected || youredead !== youredeadexpected) begin // check result

$display("Error: inputs = %b", direction);

$display(" state = %s", dut.r1.state.name);

$display(" outputs = %b %b (%b %b expected)",

youwon, youredead, youwonexpected, youredeadexpected);

errors = errors + 1;

end

vectornum = vectornum + 1;

if (testvectors[vectornum] === 6'bx) begin

$display("%d tests completed with %d errors", vectornum,

errors);

$stop;

end

end

endmodule

run 500

# To travel in 0010 direction. Currently in room CC. Sword=0 Dead=0, Won=0

# To travel in 1000 direction. Currently in room TT. Sword=0 Dead=0, Won=0

# To travel in 0010 direction. Currently in room CC. Sword=0 Dead=0, Won=0

# To travel in 0100 direction. Currently in room TT. Sword=0 Dead=0, Won=0

# To travel in 0001 direction. Currently in room RR. Sword=0 Dead=0, Won=0

# To travel in 0100 direction. Currently in room TT. Sword=0 Dead=0, Won=0

# To travel in 0010 direction. Currently in room RR. Sword=0 Dead=0, Won=0

# To travel in 0010 direction. Currently in room DD. Sword=0 Dead=0, Won=0

# To travel in 0010 direction. Currently in room GG. Sword=0 Dead=1, Won=0

# To travel in 0010 direction. Currently in room GG. Sword=0 Dead=1, Won=0

# To travel in 0010 direction. Currently in room CC. Sword=0 Dead=0, Won=0

# To travel in 1000 direction. Currently in room TT. Sword=0 Dead=0, Won=0

# To travel in 0010 direction. Currently in room CC. Sword=0 Dead=0, Won=0

# To travel in 0100 direction. Currently in room TT. Sword=0 Dead=0, Won=0

# To travel in 1000 direction. Currently in room RR. Sword=0 Dead=0, Won=0

# To travel in 0100 direction. Currently in room SSS. Sword=0 Dead=0, Won=0

# To travel in 0010 direction. Currently in room SSS. Sword=1 Dead=0, Won=0

# To travel in 0010 direction. Currently in room RR. Sword=1 Dead=0, Won=0

# To travel in 0010 direction. Currently in room DD. Sword=1 Dead=0, Won=0

# To travel in 0010 direction. Currently in room VV. Sword=1 Dead=0, Won=1

# To travel in 1000 direction. Currently in room VV. Sword=1 Dead=0, Won=1

#         21 tests completed with          0 errors

# ** Note: $stop    : C:/Users/spjut/Documents/E85 F17/Lab_4_sol_res/testbench.sv(47)

#    Time: 235 ns  Iteration: 1  Instance: /testbench

# Break in Module testbench at C:/Users/spjut/Documents/E85 F17/Lab_4_sol_res/testbench.sv line 47