Lab 6: Music Keyboard SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- [2] Schematic

- [3] Behavioral SystemVerilog code

module lab5_sol(input  logic       clk, // 50 MHz

input  logic       reset,

input  logic [3:0] key,

output logic       speaker);

logic [23:0] count;

logic [7:0]  inc;

// choose note by   counter appropriately

always_comb

case(key)

4’b0111: inc = 8’d88;  // C

4’b1011: inc = 8’d111; // E

4’b1101: inc = 8’d132; // G

4’b1110: inc = 8’d148; // A

default: inc = 0;      // no note

endcase

// counter for digitally controlled oscillator

always_ff @(posedge clk, posedge reset)

if (reset) count <= 0;

else       count <= count + inc;

assign speaker = count[23];

endmodule

- [2 for agreement within 1%, 1 for agreement within 10%] Table of actual output frequencies

| Key | Note | Desired Frequency (Hz) | Actual Frequency (Hz) |
| --- | --- | --- | --- |
| KEY3 | C | 261.6 | 262.3 |
| KEY2 | E | 329.6 | 330.8 |
| KEY1 | G | 392 | 393.4 |
| KEY0 | A | 440 | 441.1 |

- [2] Does your system play music? YES.  Ring around the rosie.

- [1XC] Extra credit, if applicable

If you have suggestions for further improvements of this lab, you’re welcome to include them at the end of your lab.