Lab 8: Simon Game SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- [2] Schematic.  Remember that pins on adjacent side of pushbutton connect when the button is pressed.  Pins on opposite sides are always connected. Any value between 330 Ω and 75 Ω should work for the LEDs. The switch pulldown value is also approximate.

- [3] C code

// lab7.c

// Josh Brake

// 1/8/2020

// Uses direct register writes instead of EasyREDVIO library

#include <stdio.h>

extern void delay(int delay_ms);

#define LED0 4      // GPIO 4

#define LED1 2      // GPIO 2

#define LED2 5      // GPIO 5

#define BUTTON0 23  // GPIO 23

#define BUTTON1 22  // GPIO 22

#define N 500000

#define L 12

char sequence[L] = "GRGRGRRGGRRG";

char cur_level_sequence[L] = "000000000000";

// Output enable register

volatile unsigned long * GPIO0_output_en = (unsigned long*) 0x10012008;

// Output XOR register (to configure output as active high)

volatile unsigned long * GPIO0_out_xor = (unsigned long*) 0x10012040;

// Function select register

volatile unsigned long * GPIO0_iof_en = (unsigned long*) 0x10012038;

// Output value register

volatile unsigned long * GPIO0_output_val = (unsigned long*) 0x1001200C;

// Input value register

volatile unsigned long * GPIO0_input_val = (unsigned long*) 0x10012000;

// Input enable register

volatile unsigned long * GPIO0_input_en = (unsigned long*) 0x10012004;

void setup()

{

// Configure LED outputs

*GPIO0_output_en |= (1 << LED0) | (1 << LED1) | (1 << LED2);

// *GPIO0_out_xor |= (1 << LED0) | (1 << LED1);

*GPIO0_iof_en &= ~((1 << LED0) | (1 << LED1) | (1 << LED2));

// Configure button inputs

*GPIO0_input_en |= (1 << BUTTON0) | (1 << BUTTON1);

}

void blink_led(int led_num, int delay_len)

{

*GPIO0_output_val |= (1 << led_num);

delay(delay_len);

*GPIO0_output_val &= ~(1 << led_num);

delay(delay_len);

}

void digitalWrite(int led_num, int value)

{

if(value == 1)

{

*GPIO0_output_val |= (1 << led_num);

}

else if (value == 0)

{

*GPIO0_output_val &= ~(1 << led_num);

}

}

int main()

{

setup();

int cur_level = 2;

int blink_dur = 300;

while(1)

{

// Fill current pattern for comparison

for(int j = 0; j < cur_level; j++)

{

if(j < cur_level)

{

cur_level_sequence[j] = sequence[j];

}

else

{

cur_level_sequence[j] = '0';

}

}

// Display length of test sequence

delay(4*blink_dur);

for(int i = 1; i <= cur_level; i++)

{

blink_led(LED2, blink_dur);

}

delay(4*blink_dur);

// Display pattern

for(int i = 1; i <= cur_level; i++)

{

char cur_led = cur_level_sequence[i-1];

if (cur_led == 'G') blink_led(LED0, blink_dur);

else if(cur_led == 'R') blink_led(LED1, blink_dur);

}

char temp_sequence[L];

// Check inputs

for(int i = 1; i <= cur_level; i++)

{

char button_press_val = '0';

while(1)

{

delay(10);

volatile unsigned long current_input = *GPIO0_input_val;

volatile unsigned long button0_pressed = (current_input >> BUTTON0) & 1;

volatile unsigned long button1_pressed = (current_input >> BUTTON1) & 1;

if(button0_pressed == 1)

{

button_press_val = 'G';

blink_led(LED0, blink_dur);

break;

}

else if(button1_pressed == 1)

{

button_press_val = 'R';

blink_led(LED1, blink_dur);

break;

}

}

temp_sequence[i-1] = button_press_val;

}

int pattern_match = 1;

for(int i = 1; i <= cur_level; i++)

{

if(temp_sequence[i-1] != cur_level_sequence[i-1])

{

pattern_match = 0;

cur_level = 2;

}

}

// Display success message pattern

if(pattern_match == 1)

{

for(int i = 0; i < 10; i++)

{

blink_led(LED2, 50);

}

}

// Next level logic

if(cur_level == L)

{

cur_level = 2;

delay(3*blink_dur);

}

else

{

cur_level++;

}

}

}

- [3] Does your Simon game work?  YES

- [1XC] Extra credit