Lab 10: Introduction to RISC-V Assembly SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- Please submit each of your code snippets: div9.s, big2little.s, and bubblesort.s.

Answers:

# div9.s

# sarah.harris@unlv.edu

# October 2021

# Description: a0 holds a positive integer. If it is divisible by 9,

# a0 is 1 after the code completes. Otherwise, a0 is 0.

#

test:    addi a0, zero, 25       # test case

check:   beq  a0, zero, isdiv9

blt  a0, zero, notdiv9

addi a0, a0, -9         # a0 = a0 - 9

j    check

notdiv9: addi a0, zero, 0        # a0 = 0 (not divisible by 9)

j    done

isdiv9:  addi a0, zero, 1        # a0 = 1 (is divisible by 9)

done:

# big2little.s

initarray:

addi a0, zero, 0x300

addi t0, zero, 32     # number of bytes

addi t1, zero, 0      # index

lui  t2, 0xABCDE      # t2 = 0xABCDE123 (random value)

addi t2, t2, 0x123

L1:

beq  t0, t1, doneinit

add  t3, a0, t1       # t3 = address to write to

sw   t2, 0(t3)        # [t3] = t2

addi t1, t1, 4        # t1 += 4 (address of next word)

slli t2, t2, 8        # change value in t2

add  t2, t2, t1

j    L1               # repeat loop

doneinit:

addi a0, zero, 0x300

addi t0, zero, 0x320  # just past last byte

L2:

beq  a0, t0, done     # branch when at end of 8 words

lb   t2, 0(a0)        # t2 = byte 0

lb   t3, 1(a0)        # t3 = byte 1

lb   t4, 2(a0)        # t4 = byte 2

lb   t5, 3(a0)        # t5 = byte 3

sb   t2, 3(a0)        # byte 3 = t2

sb   t3, 2(a0)        # byte 2 = t3

sb   t4, 1(a0)        # byte 1 = t4

sb   t5, 0(a0)        # byte 0 = t5

addi a0, a0, 4        # increment by 1 word (4 bytes)

j    L2               # repeat loop

done:

# bubblesort.c

int tmp, swapped = 1; // s0 = tmp, s1 = swapped

while (swapped) {

swapped = 0;

for (i=0; i<9; i++)

if (sortarray[i] > sortarray[i+1]) {

tmp = sortarray[i];

sortarray[i] = sortarray[i+1];

sortarray[i+1] = tmp;

swapped = 1;

}

}

# bubblesort.s

initarray:

addi a0, zero, 0x400

addi t0, zero, 89

sw   t0, 0(a0)

addi t0, zero, 63

sw   t0, 4(a0)

addi t0, zero, -55

sw   t0, 8(a0)

addi t0, zero, -107

sw   t0, 0xC(a0)

addi t0, zero, 42

sw   t0, 0x10(a0)

addi t0, zero, 98

sw   t0, 0x14(a0)

addi t0, zero, -425

sw   t0, 0x18(a0)

addi t0, zero, 203

sw   t0, 0x1C(a0)

addi t0, zero, 0

sw   t0, 0x20(a0)

addi t0, zero, 303

sw   t0, 0x24(a0)

# s0 = tmp, s1 = swapped

bubblesort:

addi a0, zero, 0x400  # base address of array

addi t0, a0, 36       # just past second to last element of array

addi s1, zero, 1      # swapped = 1

L1:

beq  s1, zero, done   # if (swapped == 0), done

addi s1, zero, 0      # swapped = 0

addi t3, a0, 0        # t3 = base address of sortarray[]

L2:

bge  t3, t0, L1       # if i >= 9, end of for loop

lw   s0, 0(t3)        # tmp = sortarray[i]

lw   t1, 4(t3)        # t1 = sortarry[i+1]

bge  t1, s0, L3       # if sa[i+1] >= sa[i], continue

sw   t1, 0(t3)        # sortarray[i] = sortarray[i+1]

sw   s0, 4(t3)        # sortarray[i+1] = tmp

addi s1, zero, 1      # swapped = 1

L3:

addi t3, t3, 4        # t3 = address of next element in array

j    L2

done:

addi zero, zero, 0    # nop – to be able to set breakpoint here

Please indicate any bugs you found in this lab manual, or any suggestions you would have to improve the lab.