Lab 11: Functions in RISC-V Assembly SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- Your code: div9Function.s, bubblesortFunction.s, gcd.s, greet.c, and greet.s.

# div9Function.s

# test cases

test:    addi a0, zero, 15       # div9(15)

jal  div9

addi a0, zero, 81       # div9(81)

jal  div9

nop

div9:    beq  a0, zero, isdiv9

blt  a0, zero, notdiv9

addi a0, a0, -9         # a0 = a0 - 9

j    div9

notdiv9: addi a0, zero, 0        # a0 = 0 (not divisible by 9)

j    done

isdiv9:  addi a0, zero, 1        # a0 = 1 (is divisible by 9)

done:    jr   ra                 # return

# bubblesortFunction.s

# Test case: sortarray = {-15,42,73,19,-8,24,16,-2,99,-78,-21,23,-88,49,-101}

# When the code completes, sortarray is:

# {-101,-88,-78,-21,-15,-8,-2,16,19,23,24,42,49,73,99}

initarray:

addi a0, zero, 0x400

addi t0, zero, -15

sw   t0, 0(a0)

addi t0, zero, 42

sw   t0, 4(a0)

addi t0, zero, 73

sw   t0, 8(a0)

addi t0, zero, 19

sw   t0, 0xC(a0)

addi t0, zero, -8

sw   t0, 0x10(a0)

addi t0, zero, 24

sw   t0, 0x14(a0)

addi t0, zero, 16

sw   t0, 0x18(a0)

addi t0, zero, -2

sw   t0, 0x1C(a0)

addi t0, zero, 99

sw   t0, 0x20(a0)

addi t0, zero, -78

sw   t0, 0x24(a0)

addi t0, zero, -21

sw   t0, 0x28(a0)

addi t0, zero, 23

sw   t0, 0x2C(a0)

addi t0, zero, -88

sw   t0, 0x30(a0)

addi t0, zero, 49

sw   t0, 0x34(a0)

addi t0, zero, -101

sw   t0, 0x38(a0)

addi a1, zero, 15

jal  bubblesort

nop

# s0 = tmp, s1 = swapped

bubblesort:

addi a1, a1, -1       # a1 = size-1W

slli t0, a1, 2        # t0 = (size-1) * 4

add  t0, a0, t0       # just past second to last element of array

addi s1, zero, 1      # swapped = 1

L1:

beq  s1, zero, done   # if (swapped == 0), done

addi s1, zero, 0      # swapped = 0

addi t3, a0, 0        # t3 = base address of sortarray[]

L2:

bge  t3, t0, L1       # if i >= (size-1), end of for loop

lw   s0, 0(t3)        # tmp = sortarray[i]

lw   t1, 4(t3)        # t1 = sortarry[i+1]

bge  t1, s0, L3       # if sa[i+1] >= sa[i], continue

sw   t1, 0(t3)        # sortarray[i] = sortarray[i+1]

sw   s0, 4(t3)        # sortarray[i+1] = tmp

addi s1, zero, 1      # swapped = 1

L3:

addi t3, t3, 4        # t3 = address of next element in array

j    L2

done:

jr   ra               # return

# gcd.s

# Algorithm:

#  int tmp;

#  if (n1 > n2) {

#    tmp = n2;

#    n2 = n1;

#    n1 = tmp;

#  }

#  while (n2 != 0 & n1 != 0) {

#    tmp = n2 % n1;

#    n2 = n1;

#    n1 = tmp;

#  }

#  if (n2 == 0) return n1;

#  else         return n2;

test:

addi a0, zero, 25     # gcd(25,15)

addi a1, zero, 15

jal  gcd

addi a0, zero, 64     # gcd(64,96)

addi a1, zero, 96

jal  gcd

addi a0, zero, 71     # gcd(71,9)

addi a1, zero, 9

jal  gcd

nop

# a1 should hold larger value

gcd: bge  a1, a0, L1    # if a1 < a0, swap a0 and a1

addi t0, a1, 0     # a1 holds max

addi a1, a0, 0     # a0 holds min

addi a0, t0, 0

L1:  beq  a1, zero, L2  # if a1 or a0 is 0, done

beq  a0, zero, L2

rem  t0, a1, a0

addi a1, a0, 0     # a1 = a0 (new max)

addi a0, t0, 0     # a0 = remainder (new min)

j    L1

L2:  beq  a1, zero, L3  # if a1 != 0, return a1

addi a0, a1, 0     # return a1

L3:  jr   ra            # return

# greet.s

addi a0, zero, 7        # greet(7) = 21

jal  greet

addi a0, zero, 10       # greet(10) = 45

jal  greet

nop

greet:

addi sp, sp, -8    # make room for two words on stack

sw   ra, 4(sp)     # save ra on stack

sw   a0, 0(sp)     # save n on stack

addi t0, zero, 2   # t0 = 1

bne  a0, t0, else  # if n != 1, do else

addi a0, zero, 0   # return 0

jr  ra

else:

addi a0, a0, -1    # n = n-1

jal  greet

lw   ra, 4(sp)     # restore registers from stack

lw   t0, 0(sp)     # t0 = n (formerly a0)

addi sp, sp, 8     # restore sp

addi t0, t0, -1    # t0 = n-1

add  a0, a0, t0    # return (n-1) + greet(n-1)

jr   ra

- Screenshots of a0 from the Venus simulator after making the following function calls:

div9(15);

div9(81);

gcd(25, 15);

gcd(64, 96);

gcd(71, 9);

greet(7);

greet(10);

- A screenshot of memory addresses 0x400 - 0x438 when bubblesort() is called with the following 15-element array, sortarray, as an argument, with the array’s base address being at 0x400. Be sure to include code that initializes the array before the function call.

sortarray[15]={-15,42,73,19,-8,24,16,-2,99,-78,-21,23,-88,49,-101};

Please indicate any bugs you found in this lab manual, or any suggestions you would have to improve the lab.