Lab 12: Airbag Trigger SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- [3] Your assembly language implementation.

// lab9asm.c

// David_Harris@hmc.edu 29 March 2017

#include "EasyNucleoIO.h"

// prototype for asm file

void triggerCheck(void);

int main(void) {

EasyNucleoIOInit();

pinMode(0, INPUT);

pinMode(1, INPUT);

pinMode(2, OUTPUT);

triggerCheck();

}

; triggerCheck.s

; David_Harris@hmc.edu 29 March 2017

;	volatile unsigned long *GPIOA_IDR = (unsigned long*)0x48000010;

;	volatile unsigned long *GPIOA_ODR = (unsigned long*)0x48000014;

;	while(1) { // assume it is ok to set all other output bits to 0

;		if ((*GPIOA_IDR & INMASK) == INMASK) *GPIOA_ODR = D2MASK;

;		else                                 *GPIOA_ODR = 0;

;	}

AREA |.text|, CODE, READONLY 	; define this file as code

EXPORT triggerCheck			; declare triggerCheck to be called externally

GPIOA_IDR EQU 0x48000010		; define constant address of port

GPIOA_ODR EQU 0x48000014		; define constant address of port

INMASK    EQU 0x600			; 1 in bits 9 and 10 for PA9 and PA10 (D1, D0)

D2MASK	  EQU 0x1000			; 1 in bit 12 for PA12 (D2)

triggerCheck

LDR R0, =GPIOA_IDR			; put addresses in registers

LDR R1, =GPIOA_ODR

LDR R2, =INMASK			; put constants in registers

LDR R3, =D2MASK

LDR R4, =0

while

LDR R5, [R0]				; R5 = *GIOA_IDR

ANDS R5, R2				; R5 = R5 & INMASK

CMP R5, R2				; R5 = INMASK?  Both inputs true?

BNE else

STR R3, [R1]				; yes: *GPIO_ODR = D2MASK

B while

else

STR R4, [R1]				; no: *GPIO_ODR = 0

B while

ALIGN 				; make sure code ends on word boundary

END

- [3] Your optimized C implementation.  (here is one possibility)

// lab9opt.c

// David_Harris@hmc.edu 29 March 2017

#include "EasyNucleoIO.h"

#define D0MASK (1 << 10)

#define D1MASK (1 << 9)

#define INMASK (D0MASK | D1MASK)

#define D2MASK (1 << 12)

void triggerCheck(void) {

volatile unsigned long *GPIOA_IDR = (unsigned long*)0x48000010;

volatile unsigned long *GPIOA_ODR = (unsigned long*)0x48000014;

while(1) { // assume it is ok to set all other output bits to 0

if ((*GPIOA_IDR & INMASK) == INMASK) *GPIOA_ODR = D2MASK;

else                                 *GPIOA_ODR = 0;

}

}

int main(void) {

EasyNucleoIOInit();

pinMode(0, INPUT);

pinMode(1, INPUT);

pinMode(2, OUTPUT);

triggerCheck();

}

- [3] A table of instruction count and average, max, and standard deviation of latency for each of the three implementations.

(these are my measurements.  Yours will show some statistical variation, but should be in this general range)

|  | Baseline | Assembly | Optimized C |
| --- | --- | --- | --- |
| Greatest Instruction Count | 130 | 6 | 10 |
| Avg Latency (s) | 27.072 | 1.515 | 2.197 |
| Max Latency (s) | 39.528 | 3.132 | 3.146 |
| Stdev Latency (s) | 11.988 | 0.830 | 1.329 |

The instruction count derivation is shown below.

Baseline

triggerCheck exectues 20 instructions (excluding setup) + 2 calls to digitalRead and one to digitalWrite.

digitalRead executes 13 instructions + 1 call to pinToReg and 1 to pinToOffset.

digitalWrite executes 18 instructions + 1 call to pinToReg and 1 to pinToOffset.

pinToReg executes 10 instructions (to return PORTB).

pinToOffset executes 12 instructions.

Hence, triggerCheck executes 20 + 2* (13 + 10 + 12) + (18 + 10 + 12) = 130 instructions.

Assembly

The while loop executes 6 instructions.

26: while

0x0800016C 4C08      LDR      r4,[pc,#32]  ; @0x08000190

27:         LDR R5, [R0]               ; R5 = *GIOA_IDR

0x0800016E 6805      LDR      r5,[r0,#0x00]

28:         ANDS R5, R2                ; R5 = R5 & INMASK

0x08000170 4015      ANDS     r5,r5,r2

29:         CMP R5, R2			; R5 = INMASK?  Both inputs true?

0x08000172 4295      CMP      r5,r2

30:         BNE else

0x08000174 D101      BNE      0x0800017A

31:         STR R3, [R1]               ; yes: *GPIO_ODR = D2MASK

0x08000176 600B      STR      r3,[r1,#0x00]

32:         B while

33: else

0x08000178 E7F9      B        0x0800016E

34:         STR R5, [R1]                 ; no: *GPIO_ODR = 0

0x0800017A 600D      STR      r5,[r1,#0x00]

35:         B while

0x0800017C E7F7      B        0x0800016E

Optimized C

The while loop executes 10 instructions in the worst case.

13:         while(1) { // assume it is ok to set all other output bits to 0

0x08000374 E00B      B        0x0800038E

14:         if ((*GPIOA_IDR & INMASK) == INMASK) *GPIOA_ODR = D2MASK;

0x08000376 680A      LDR      r2,[r1,#0x00]

0x08000378 2303      MOVS     r3,#0x03

0x0800037A 025B      LSLS     r3,r3,#9

0x0800037C 401A      ANDS     r2,r2,r3

0x0800037E 429A      CMP      r2,r3

0x08000380 D103      BNE      0x0800038A

0x08000382 2201      MOVS     r2,#0x01

0x08000384 0312      LSLS     r2,r2,#12

0x08000386 6002      STR      r2,[r0,#0x00]

0x08000388 E001      B        0x0800038E

15:          else    *GPIOA_ODR = 0;

0x0800038A 2200      MOVS     r2,#0x00

0x0800038C 6002      STR      r2,[r0,#0x00]

0x0800038E E7F2      B        0x08000376

If you have suggestions for further improvements of this lab, you’re welcome to include them at the end of your lab.