Lab 11: Single-Cycle Processor SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- A marked up version of Figure 1 (single-cycle processor) showing the needed modifications (lui and xor)

Answer:

Figure 1: RISC-V single-cycle processor

- A marked up version of Figures 2 and 3 (if modified) showing the needed modifications for the additional instructions (lui and xor)

Answer:

Figure 2: RISC-V single-cycle processor control unit modified to support xor and lui

Figure 3: ALU modified to support xor

- Amended Main Decoder, ALU Decoder, and ImmSrc truth tables to support lui and xor

Answer:

Table 1. Main Decoder Truth Table

| Instruction | Opcode | RegWrite | ImmSrc | ALUSrcA | ALUSrcB | MemWrite | ResultSrc | Branch | ALUOp | Jump |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lw | 0000011 | 1 | 000 | 0 | 1 | 0 | 01 | 0 | 00 | 0 |
| sw | 0100011 | 0 | 001 | 0 | 1 | 1 | xx | 0 | 00 | 0 |
| R-type | 0110011 | 1 | xxx | 0 | 0 | 0 | 00 | 0 | 10 | 0 |
| beq | 1100011 | 0 | 010 | 0 | 0 | 0 | xx | 1 | 01 | 0 |
| I-type ALU | 0010011 | 1 | 000 | 0 | 0 | 0 | 00 | 0 | 10 | 0 |
| jal | 1101111 | 1 | 011 | x | x | 0 | 10 | 0 | xx | 1 |
| lui | 0110111 | 1 | 100 | 1 | 1 | 0 | 00 | 0 | 00 | 0 |

Table 2: ALU Decoder Truth Table to support xor

| ALUOp | funct3 | {op5, funct75} | ALUControl | Instruction |
| --- | --- | --- | --- | --- |
| 00 | x | x | 000 (add) | lw, sw |
| 01 | x | x | 001 (subtract) | beq |
| 10 | 000 | 00, 01, 10 | 000 (add) | add, addi |
|  | 000 | 11 | 001 (subtract) | sub |
|  | 010 | x | 101 (set less than) | slt, slti |
|  | 100 | x | 100 (xor) | xor, xori |
|  | 110 | x | 011 (or) | or, ori |
|  | 111 | x | 010 (and) | and, andi |

Table 3.  ImmSrc encoding to support lui

| ImmSrc | ImmExt | Type | Description |
| --- | --- | --- | --- |
| 000 | {{20{Instr[31]}}, Instr[31:20]} | I | 12-bit signed immediate |
| 001 | {{20{Instr[31]}}, Instr[31:25], Instr[11:7]} | S | 12-bit signed immediate |
| 010 | {{20{Instr[31]}}, Instr[7], Instr[30:25], Instr[11:8], 1’b0} | B | 13-bit signed immediate |
| 011 | {{12{Instr[31]}}, Instr[19:12], Instr[20], Instr[30:21], 1’b0} | J | 21-bit signed immediate |
| 100 | {Instr[31:12], 12’b0} | U | 20-bit signed immediate |

- Amended SystemVerilog code to add support for lui and xor

Answer:

module testbench();

logic        clk;

logic        reset;

logic [31:0] WriteData, DataAdr;

logic        MemWrite;

// instantiate device to be tested

top dut(clk, reset, WriteData, DataAdr, MemWrite);

// initialize test

initial

begin

reset <= 1; # 22; reset <= 0;

end

// generate clock to sequence tests

always

begin

clk <= 1; # 5; clk <= 0; # 5;

end

// check results

always @(negedge clk)

begin

if(MemWrite) begin

if(DataAdr === 108 & WriteData === 32'hABCDE7D5) begin

$display("Simulation succeeded");

$stop;

end else if (DataAdr !== 96) begin

$display("Simulation failed");

$stop;

end

end

end

endmodule

module top(input  logic        clk, reset,

output logic [31:0] WriteData, DataAdr,

output logic        MemWrite);

logic [31:0] PC, Instr, ReadData;

// instantiate processor and memories

riscvsingle rvsingle(clk, reset, PC, Instr, MemWrite, DataAdr,

WriteData, ReadData);

imem imem(PC, Instr);

dmem dmem(clk, MemWrite, DataAdr, WriteData, ReadData);

endmodule

module riscvsingle(input  logic        clk, reset,

output logic [31:0] PC,

input  logic [31:0] Instr,

output logic        MemWrite,

output logic [31:0] ALUResult, WriteData,

input  logic [31:0] ReadData);

logic       ALUSrcA, ALUSrcB;     // lui

logic       RegWrite, Jump, Zero;

logic [1:0] ResultSrc;

logic [2:0] ImmSrc;               // lui

logic [2:0] ALUControl;

controller c(Instr[6:0], Instr[14:12], Instr[30], Zero,

ResultSrc, MemWrite, PCSrc,

ALUSrcA, ALUSrcB, RegWrite, Jump,

ImmSrc, ALUControl);

datapath dp(clk, reset, ResultSrc, PCSrc,

ALUSrcA, ALUSrcB, RegWrite,

ImmSrc, ALUControl,

Zero, PC, Instr,

ALUResult, WriteData, ReadData);

endmodule

module controller(input  logic [6:0] op,

input  logic [2:0] funct3,

input  logic       funct7b5,

input  logic       Zero,

output logic [1:0] ResultSrc,

output logic       MemWrite,

output logic       PCSrc,

output logic       ALUSrcA, ALUSrcB,  // lui

output logic       RegWrite, Jump,

output logic [2:0] ImmSrc,            // lui: 3 bits

output logic [2:0] ALUControl);

logic [1:0] ALUOp;

logic       Branch;

maindec md(op, ResultSrc, MemWrite, Branch,

ALUSrcA, ALUSrcB, RegWrite, Jump, ImmSrc, ALUOp);

aludec  ad(op[5], funct3, funct7b5, ALUOp, ALUControl);

assign PCSrc = Branch & Zero | Jump;

endmodule

module maindec(input  logic [6:0] op,

output logic [1:0] ResultSrc,

output logic       MemWrite,

output logic       Branch,

output logic       ALUSrcA, ALUSrcB, // lui

output logic       RegWrite, Jump,

output logic [2:0] ImmSrc,   // lui: 3 bits

output logic [1:0] ALUOp);

logic [12:0] controls;

assign {RegWrite, ImmSrc, ALUSrcA, ALUSrcB, MemWrite,

ResultSrc, Branch, ALUOp, Jump} = controls;

always_comb

case(op)

// RegWrite_ImmSrc_ALUSrcA_ALUSrcB_MemWrite_ResultSrc_Branch_ALUOp_Jump

7'b0000011: controls = 13'b1_000_0_1_0_01_0_00_0; // lw

7'b0100011: controls = 13'b0_001_0_1_1_00_0_00_0; // sw

7'b0110011: controls = 13'b1_xxx_0_0_0_00_0_10_0; // R-type

7'b1100011: controls = 13'b0_010_0_0_0_00_1_01_0; // beq

7'b0010011: controls = 13'b1_000_0_1_0_00_0_10_0; // I-type ALU

7'b1101111: controls = 13'b1_011_0_1_0_10_0_00_1; // jal

7'b0110111: controls = 13'b1_100_1_1_0_00_0_00_0; // lui

default:    controls = 13'bx_xxx_x_x_x_xx_x_xx_x; // non-implemented

endcase

endmodule

module aludec(input  logic       opb5,

input  logic [2:0] funct3,

input  logic       funct7b5,

input  logic [1:0] ALUOp,

output logic [2:0] ALUControl);

logic  RtypeSub;

assign RtypeSub = funct7b5 & opb5;  // TRUE for R-type subtract instruction

always_comb

case(ALUOp)

2'b00:                ALUControl = 3'b000; // addition

2'b01:                ALUControl = 3'b001; // subtraction

default: case(funct3) // R-type or I-type ALU

3'b000:  if (RtypeSub)

ALUControl = 3'b001; // sub

else

ALUControl = 3'b000; // add, addi

3'b010:    ALUControl = 3'b101; // slt, slti

3'b110:    ALUControl = 3'b011; // or, ori

3'b111:    ALUControl = 3'b010; // and, andi

3'b100:    ALUControl = 3'b100; // xor, xori

default:   ALUControl = 3'bxxx; // ???

endcase

endcase

endmodule

module datapath(input  logic        clk, reset,

input  logic [1:0]  ResultSrc,

input  logic        PCSrc,

input  logic        ALUSrcA, ALUSrcB, // lui

input  logic        RegWrite,

input  logic [2:0]  ImmSrc,  // lui: 3-bit

input  logic [2:0]  ALUControl,

output logic        Zero,

output logic [31:0] PC,

input  logic [31:0] Instr,

output logic [31:0] ALUResult, WriteData,

input  logic [31:0] ReadData);

logic [31:0] PCNext, PCPlus4, PCTarget;

logic [31:0] ImmExt;

logic [31:0] SrcA, SrcB;

logic [31:0] A;  // lui

logic [31:0] Result;

// next PC logic

flopr #(32) pcreg(clk, reset, PCNext, PC);

adder       pcadd4(PC, 32'd4, PCPlus4);

adder       pcaddbranch(PC, ImmExt, PCTarget);

mux2 #(32)  pcmux(PCPlus4, PCTarget, PCSrc, PCNext);

// register file logic

regfile     rf(clk, RegWrite, Instr[19:15], Instr[24:20],

Instr[11:7], Result, A, WriteData);

extend      ext(Instr[31:7], ImmSrc, ImmExt);

// ALU logic

mux2 #(32)  srcamux(A, 32'b0, ALUSrcA, SrcA); // lui

mux2 #(32)  srcbmux(WriteData, ImmExt, ALUSrcB, SrcB);

alu         alu(SrcA, SrcB, ALUControl, ALUResult, Zero);

mux3 #(32)  resultmux(ALUResult, ReadData, PCPlus4, ResultSrc, Result);

endmodule

module regfile(input  logic        clk,

input  logic        we3,

input  logic [ 4:0] a1, a2, a3,

input  logic [31:0] wd3,

output logic [31:0] rd1, rd2);

logic [31:0] rf[31:0];

// three ported register file

// read two ports combinationally (A1/RD1, A2/RD2)

// write third port on rising edge of clock (A3/WD3/WE3)

// register 0 hardwired to 0

always_ff @(posedge clk)

if (we3) rf[a3] <= wd3;

assign rd1 = (a1 != 0) ? rf[a1] : 0;

assign rd2 = (a2 != 0) ? rf[a2] : 0;

endmodule

module adder(input  [31:0] a, b,

output [31:0] y);

assign y = a + b;

endmodule

module extend(input  logic [31:7] instr,

input  logic [2:0]  immsrc,   // lui

output logic [31:0] immext);

always_comb

case(immsrc)

// I-type

3'b000:   immext = {{20{instr[31]}}, instr[31:20]};

// S-type (stores)

3'b001:   immext = {{20{instr[31]}}, instr[31:25], instr[11:7]};

// B-type (branches)

3'b010:   immext = {{20{instr[31]}}, instr[7], instr[30:25], instr[11:8], 1'b0};

// J-type (jal)

3'b011:   immext = {{12{instr[31]}}, instr[19:12], instr[20], instr[30:21], 1'b0};

// U-type (lui)

3'b100:   immext = {instr[31:12], 12'b0};

default:  immext = 32'bx; // undefined

endcase

endmodule

module flopr #(parameter WIDTH = 8)

(input  logic             clk, reset,

input  logic [WIDTH-1:0] d,

output logic [WIDTH-1:0] q);

always_ff @(posedge clk, posedge reset)

if (reset) q <= 0;

else       q <= d;

endmodule

module mux2 #(parameter WIDTH = 8)

(input  logic [WIDTH-1:0] d0, d1,

input  logic             s,

output logic [WIDTH-1:0] y);

assign y = s ? d1 : d0;

endmodule

module mux3 #(parameter WIDTH = 8)

(input  logic [WIDTH-1:0] d0, d1, d2,

input  logic [1:0]       s,

output logic [WIDTH-1:0] y);

assign y = s[1] ? d2 : (s[0] ? d1 : d0);

endmodule

module imem(input  logic [31:0] a,

output logic [31:0] rd);

logic [31:0] RAM[63:0];

initial

$readmemh("riscvtest.txt",RAM);

assign rd = RAM[a[31:2]]; // word aligned

endmodule

module dmem(input  logic        clk, we,

input  logic [31:0] a, wd,

output logic [31:0] rd);

logic [31:0] RAM[63:0];

assign rd = RAM[a[31:2]]; // word aligned

always_ff @(posedge clk)

if (we) RAM[a[31:2]] <= wd;

endmodule

module alu(input  logic [31:0] a, b,

input  logic [2:0]  alucontrol,

output logic [31:0] result,

output logic        zero);

logic [31:0] condinvb, sum;

logic        v;              // overflow

logic        isAddSub;       // true when is add or subtract operation

assign condinvb = alucontrol[0] ? ~b : b;

assign sum = a + condinvb + alucontrol[0];

assign isAddSub = ~alucontrol[2] & ~alucontrol[1] |

~alucontrol[1] & alucontrol[0];

always_comb

case (alucontrol)

3'b000:  result = sum;         // add

3'b001:  result = sum;         // subtract

3'b010:  result = a & b;       // and

3'b011:  result = a | b;       // or

3'b100:  result = a ^ b;       // xor

3'b101:  result = sum[31] ^ v; // slt

3'b110:  result = a << b[4:0]; // sll

3'b111:  result = a >> b[4:0]; // srl

default: result = 32'bx;

endcase

assign zero = (result == 32'b0);

assign v = ~(alucontrol[0] ^ a[31] ^ b[31]) & (a[31] ^ sum[31]) & isAddSub;

endmodule

- Modified test program that uses lui and xor

Answer:

Answers will vary – but the modified test program must fail if xor or lui doesn’t work correctly.

# If successful, it should write the value 0xABCDE7D5 to address 108

#       RISC-V Assembly         Description               Address   Machine

main:   addi x2, x0, 5          # x2 = 5                  0         00500113

addi x3, x0, 12         # x3 = 12                 4         00C00193

addi x7, x3, -9         # x7 = (12 - 9) = 3       8         FF718393

or   x4, x7, x2         # x4 = (3 OR 5) = 7       C         0023E233

and  x5, x3, x4         # x5 = (12 AND 7) = 4     10        0041F2B3

add  x5, x5, x4         # x5 = (4 + 7) = 11       14        004282B3

beq  x5, x7, end        # shouldn't be taken      18        02728863

slt  x4, x3, x4         # x4 = (12 < 7) = 0       1C        0041A233

beq  x4, x0, around     # should be taken         20        00020463

addi x5, x0, 0          # shouldn't happen        24        00000293

around: slt  x4, x7, x2         # x4 = (3 < 5)  = 1       28        0023A233

add  x7, x4, x5         # x7 = (1 + 11) = 12      2C        005203B3

sub  x7, x7, x2         # x7 = (12 - 5) = 7       30        402383B3

sw   x7, 84(x3)         # [96] = 7                34        0471AA23

lw   x2, 96(x0)         # x2 = [96] = 7           38        06002103

add  x9, x2, x5         # x9 = (7 + 11) = 18      3C        005104B3

lui  x4, 0xABCDE        # ***x4 = 0xABCDE000      40        ABCDE237

addi x4, x4, 0x7CC      # ***x4 = 0xABCDE7CC      44        7CC20213

jal  x3, end            # jump to end, x3 = 0x4C  48        008001EF

addi x2, x0, 1          # shouldn't happen        4C        00100113

end:    add  x2, x2, x9         # x2 = (7 + 18)  = 25     50        00910133

xor  x2, x2, x4         # ***x2 = 0xABCDE7D5      54        00414133

sw   x2, 0x20(x3)       # mem[108] = 0xABCDE7D5   58        0221A023

done:   beq  x2, x2, done       # infinite loop           5C        00210063

- Simulation waveforms (in the order listed above: clk, reset, PC, Instr, SrcA, SrcB, ALUResult, DataAdr, WriteData, and MemWrite – all displayed in hexadecimal for ease of reading). Does your system pass your testbench? Circle or highlight the waves showing that the correct value is written to the correct address, and make sure it is legible.

Answer:

Please indicate any bugs you found in this lab manual, or any suggestions you would have to improve the lab.