Lab 11: Multicycle Processor Controller SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- Hierarchical SystemVerilog for your controller module matching the declaration given above.

module controller(input  logic       clk,

input  logic       reset,

input  logic [6:0] op,

input  logic [2:0] funct3,

input  logic       funct7b5,

input  logic       Zero,

output logic [1:0] ImmSrc,

output logic [1:0] ALUSrcA, ALUSrcB,

output logic [1:0] ResultSrc,

output logic       AdrSrc,

output logic [2:0] ALUControl,

output logic       IRWrite, PCWrite,

output logic       RegWrite, MemWrite);

logic [1:0] ALUOp;

logic       Branch, PCUpdate;

// Main FSM

mainfsm fsm(clk, reset, op,

ALUSrcA, ALUSrcB, ResultSrc, AdrSrc,

IRWrite, PCUpdate, RegWrite, MemWrite,

ALUOp, Branch);

// ALU Decoder

aludec  ad(op[5], funct3, funct7b5, ALUOp, ALUControl);

// Instruction Decoder

instrdec id(op, ImmSrc);

// Branch logic

assign PCWrite = (Branch & Zero) | PCUpdate;

endmodule

module mainfsm(input  logic         clk,

input  logic         reset,

input  logic [6:0]   op,

output logic [1:0]   ALUSrcA, ALUSrcB,

output logic [1:0]   ResultSrc,

output logic         AdrSrc,

output logic         IRWrite, PCUpdate,

output logic         RegWrite, MemWrite,

output logic [1:0]   ALUOp,

output logic         Branch);

typedef enum logic [3:0] {FETCH, DECODE, MEMADR, MEMREAD, MEMWB,

MEMWRITE,

EXECUTER, EXECUTEI, ALUWB,

BEQ, JAL, UNKNOWN} statetype;

statetype state, nextstate;

logic [14:0] controls;

// state register

always @(posedge clk or posedge reset)

if (reset) state <= FETCH;

else state <= nextstate;

// next state logic

always_comb

case(state)

FETCH:                     nextstate = DECODE;

DECODE: casez(op)

7'b0?00011:      nextstate = MEMADR;    // lw or sw

7'b0110011:      nextstate = EXECUTER;  // R-type

7'b0010011:      nextstate = EXECUTEI;  // addi

7'b1100011:      nextstate = BEQ;       // beq

7'b1101111:      nextstate = JAL;       // jal

default:         nextstate = UNKNOWN;

endcase

MEMADR:

if (op[5])               nextstate = MEMWRITE;  // sw

else                     nextstate = MEMREAD;   // lw

MEMREAD:                   nextstate = MEMWB;

EXECUTER:                  nextstate = ALUWB;

EXECUTEI:                  nextstate = ALUWB;

JAL:                       nextstate = ALUWB;

default:                   nextstate = FETCH;

endcase

// state-dependent output logic

always_comb

case(state)

FETCH: 	controls = 15'b00_10_10_0_1100_00_0;

DECODE:  	controls = 15'b01_01_00_0_0000_00_0;

MEMADR:  	controls = 15'b10_01_00_0_0000_00_0;

MEMREAD:  controls = 15'b00_00_00_1_0000_00_0;

MEMWRITE: controls = 15'b00_00_00_1_0001_00_0;

MEMWB:   	controls = 15'b00_00_01_0_0010_00_0;

EXECUTER:	controls = 15'b10_00_00_0_0000_10_0;

EXECUTEI: controls = 15'b10_01_00_0_0000_10_0;

ALUWB:    controls = 15'b00_00_00_0_0010_00_0;

BEQ:  	controls = 15'b10_00_00_0_0000_01_1;

JAL:  	controls = 15'b01_10_00_0_0100_00_0;

default: 	controls = 15'bxx_xx_xx_x_xxxx_xx_x;

endcase

assign {ALUSrcA, ALUSrcB, ResultSrc, AdrSrc, IRWrite, PCUpdate,

RegWrite, MemWrite, ALUOp, Branch} = controls;

endmodule

module aludec(input  logic       opb5,

input  logic [2:0] funct3,

input  logic       funct7b5,

input  logic [1:0] ALUOp,

output logic [2:0] ALUControl);

logic  RtypeSub;

assign RtypeSub = funct7b5 & opb5;  // TRUE for R-type subtract instruction

always_comb

case(ALUOp)

2'b00:                ALUControl = 3'b000; // addition

2'b01:                ALUControl = 3'b001; // subtraction

default: case(funct3) // R-type or I-type ALU

3'b000:  if (RtypeSub)

ALUControl = 3'b001; // sub

else

ALUControl = 3'b000; // add, addi

3'b010:    ALUControl = 3'b101; // slt, slti

3'b110:    ALUControl = 3'b011; // or, ori

3'b111:    ALUControl = 3'b010; // and, andi

default:   ALUControl = 3'bxxx; // ???

endcase

endcase

endmodule

module instrdec (input  logic [6:0] op,

output logic [1:0] ImmSrc);

always_comb

case(op)

7'b0110011: ImmSrc = 2'bxx; // R-type

7'b0010011: ImmSrc = 2'b00; // I-type ALU

7'b0000011: ImmSrc = 2'b00; // lw

7'b0100011: ImmSrc = 2'b01; // sw

7'b1100011: ImmSrc = 2'b10; // beq

7'b1101111: ImmSrc = 2'b11; // jal

default:    ImmSrc = 2'bxx; // ???

endcase

endmodule

- Does your controller pass your test vectors? YES.

Please indicate any bugs you found in this lab manual, or any suggestions you would have to improve the lab.