Lab 15: Multicycle Processor SOLUTIONS

Digital Design and Computer Architecture: RISC-V Edition (Harris & Harris, Elsevier © 2021)

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

- Diagram showing your memory, riscv, datapath, and controller block hierarchy and names of all signals between them.

Answer:

- Hierarchical SystemVerilog for your top-level processor module (and submodules) matching the declaration given in the lab.

Answer:

module top(input  logic        clk, reset,

output logic [31:0] WriteData, DataAdr,

output logic        MemWrite);

logic [31:0] ReadData;

// instantiate processor and memories

riscvmulti rvmulti(clk, reset, MemWrite, DataAdr,

WriteData, ReadData);

mem mem(clk, MemWrite, DataAdr, WriteData, ReadData);

endmodule

module riscvmulti(input  logic        clk, reset,

output logic        MemWrite,

output logic [31:0] Adr, WriteData,

input  logic [31:0] ReadData);

logic        RegWrite, jump;

logic [1:0]  ResultSrc, ImmSrc;

logic [2:0]  ALUControl;

logic 	     PCWrite;

logic 	     IRWrite;

logic [1:0]  ALUSrcA;

logic [1:0]  ALUSrcB;

logic		     AdrSrc;

logic        Zero;

logic [6:0]  op;

logic [2:0]  funct3;

logic        funct7b5;

controller c(clk, reset, op, funct3, funct7b5, Zero,

ImmSrc, ALUSrcA, ALUSrcB,

ResultSrc, AdrSrc, ALUControl,

IRWrite, PCWrite, RegWrite, MemWrite);

datapath   dp(clk, reset,

ImmSrc, ALUSrcA, ALUSrcB,

ResultSrc, AdrSrc, IRWrite, PCWrite,

RegWrite, MemWrite, ALUControl, op, funct3,

funct7b5, Zero, Adr, ReadData, WriteData);

endmodule

module controller(input  logic       clk,

input  logic       reset,

input  logic [6:0] op,

input  logic [2:0] funct3,

input  logic       funct7b5,

input  logic       Zero,

output logic [1:0] ImmSrc,

output logic [1:0] ALUSrcA, ALUSrcB,

output logic [1:0] ResultSrc,

output logic       AdrSrc,

output logic [2:0] ALUControl,

output logic       IRWrite, PCWrite,

output logic       RegWrite, MemWrite);

logic [1:0] ALUOp;

logic       Branch, PCUpdate;

// Main FSM

mainfsm fsm(clk, reset, op,

ALUSrcA, ALUSrcB, ResultSrc, AdrSrc,

IRWrite, PCUpdate, RegWrite, MemWrite,

ALUOp, Branch);

// ALU Decoder

aludec  ad(op[5], funct3, funct7b5, ALUOp, ALUControl);

// Instruction Decoder

instrdec id(op, ImmSrc);

// Branch logic

assign PCWrite = (Branch & Zero) | PCUpdate;

endmodule

module mainfsm(input  logic         clk,

input  logic         reset,

input  logic [6:0]   op,

output logic [1:0]   ALUSrcA, ALUSrcB,

output logic [1:0]   ResultSrc,

output logic         AdrSrc,

output logic         IRWrite, PCUpdate,

output logic         RegWrite, MemWrite,

output logic [1:0]   ALUOp,

output logic         Branch);

typedef enum logic [3:0] {FETCH, DECODE, MEMADR, MEMREAD, MEMWB, MEMWRITE,

EXECUTER, EXECUTEI, ALUWB,

BEQ, JAL, UNKNOWN} statetype;

statetype state, nextstate;

logic [14:0] controls;

// state register

always @(posedge clk or posedge reset)

if (reset) state <= FETCH;

else state <= nextstate;

// next state logic

always_comb

case(state)

FETCH:                     nextstate = DECODE;

DECODE: casez(op)

7'b0?00011:      nextstate = MEMADR;    // lw or sw

7'b0110011:      nextstate = EXECUTER;  // R-type

7'b0010011:      nextstate = EXECUTEI;  // addi

7'b1100011:      nextstate = BEQ;       // beq

7'b1101111:      nextstate = JAL;       // jal

default:         nextstate = UNKNOWN;

endcase

MEMADR:

if (op[5])               nextstate = MEMWRITE;  // sw

else                     nextstate = MEMREAD;   // lw

MEMREAD:                   nextstate = MEMWB;

EXECUTER:                  nextstate = ALUWB;

EXECUTEI:                  nextstate = ALUWB;

JAL:                       nextstate = ALUWB;

default:                   nextstate = FETCH;

endcase

// state-dependent output logic

always_comb

case(state)

FETCH: 	controls = 15'b00_10_10_0_1100_00_0;

DECODE:  	controls = 15'b01_01_00_0_0000_00_0;

MEMADR:  	controls = 15'b10_01_00_0_0000_00_0;

MEMREAD:  controls = 15'b00_00_00_1_0000_00_0;

MEMWRITE: controls = 15'b00_00_00_1_0001_00_0;

MEMWB:   	controls = 15'b00_00_01_0_0010_00_0;

EXECUTER:	controls = 15'b10_00_00_0_0000_10_0;

EXECUTEI: controls = 15'b10_01_00_0_0000_10_0;

ALUWB:    controls = 15'b00_00_00_0_0010_00_0;

BEQ:  	controls = 15'b10_00_00_0_0000_01_1;

JAL:  	controls = 15'b01_10_00_0_0100_00_0;

default: 	controls = 15'bxx_xx_xx_x_xxxx_xx_x;

endcase

assign {ALUSrcA, ALUSrcB, ResultSrc, AdrSrc, IRWrite, PCUpdate,

RegWrite, MemWrite, ALUOp, Branch} = controls;

endmodule

module aludec(input  logic       opb5,

input  logic [2:0] funct3,

input  logic       funct7b5,

input  logic [1:0] ALUOp,

output logic [2:0] ALUControl);

logic  RtypeSub;

assign RtypeSub = funct7b5 & opb5;  // TRUE for R-type subtract instruction

always_comb

case(ALUOp)

2'b00:                ALUControl = 3'b000; // addition

2'b01:                ALUControl = 3'b001; // subtraction

default: case(funct3) // R-type or I-type ALU

3'b000:  if (RtypeSub)

ALUControl = 3'b001; // sub

else

ALUControl = 3'b000; // add, addi

3'b010:    ALUControl = 3'b101; // slt, slti

3'b110:    ALUControl = 3'b011; // or, ori

3'b111:    ALUControl = 3'b010; // and, andi

default:   ALUControl = 3'bxxx; // ???

endcase

endcase

endmodule

module instrdec (input  logic [6:0] op,

output logic [1:0] ImmSrc);

always_comb

case(op)

7'b0110011: ImmSrc = 2'bxx; // R-type

7'b0010011: ImmSrc = 2'b00; // I-type ALU

7'b0000011: ImmSrc = 2'b00; // lw

7'b0100011: ImmSrc = 2'b01; // sw

7'b1100011: ImmSrc = 2'b10; // beq

7'b1101111: ImmSrc = 2'b11; // jal

default:    ImmSrc = 2'bxx; // ???

endcase

endmodule

module datapath(input  logic        clk, reset,

input  logic [1:0]  ImmSrc, ALUSrcA, ALUSrcB,

input  logic [1:0]  ResultSrc,

input  logic        AdrSrc,

input  logic        IRWrite, PCWrite,

input  logic        RegWrite, MemWrite,

input  logic [2:0]  alucontrol,

output logic [6:0]  op,

output logic [2:0]  funct3,

output logic        funct7b5,

output logic        Zero,

output logic [31:0] Adr,

input  logic [31:0] ReadData,

output logic [31:0] WriteData);

logic [31:0] PC, OldPC, Instr, immext, ALUResult;

logic [31:0] SrcA, SrcB, RD1, RD2, A;

logic [31:0] Result, Data, ALUOut;

// next PC logic

flopenr #(32) pcreg(clk, reset, PCWrite, Result, PC);

flopenr #(32) oldpcreg(clk, reset, IRWrite, PC, OldPC);

// memory logic

mux2    #(32) adrmux(PC, Result, AdrSrc, Adr);

flopenr #(32) ir(clk, reset, IRWrite, ReadData, Instr);

flopr   #(32) datareg(clk, reset, ReadData, Data);

// register file logic

regfile       rf(clk, RegWrite, Instr[19:15], Instr[24:20],

Instr[11:7], Result, RD1, RD2);

extend        ext(Instr[31:7], ImmSrc, immext);

flopr  #(32)  srcareg(clk, reset, RD1, A);

flopr  #(32)  wdreg(clk, reset, RD2, WriteData);

// ALU logic

mux3   #(32)  srcamux(PC, OldPC, A, ALUSrcA, SrcA);

mux3   #(32)  srcbmux(WriteData, immext, 32'd4, ALUSrcB, SrcB);

alu           alu(SrcA, SrcB, alucontrol, ALUResult, Zero);

flopr  #(32)  aluoutreg(clk, reset, ALUResult, ALUOut);

mux3   #(32)  resmux(ALUOut, Data, ALUResult, ResultSrc, Result);

// outputs to control unit

assign op       = Instr[6:0];

assign funct3   = Instr[14:12];

assign funct7b5 = Instr[30];

endmodule

module regfile(input  logic        clk,

input  logic        we3,

input  logic [ 4:0] a1, a2, a3,

input  logic [31:0] wd3,

output logic [31:0] rd1, rd2);

logic [31:0] rf[31:0];

// three ported register file

// read two ports combinationally (A1/RD1, A2/RD2)

// write third port on rising edge of clock (A3/WD3/WE3)

// register 0 hardwired to 0

always_ff @(posedge clk)

if (we3) rf[a3] <= wd3;

assign rd1 = (a1 != 0) ? rf[a1] : 0;

assign rd2 = (a2 != 0) ? rf[a2] : 0;

endmodule

module adder(input  [31:0] a, b,

output [31:0] y);

assign y = a + b;

endmodule

module extend(input  logic [31:7] instr,

input  logic [1:0]  immsrc,

output logic [31:0] immext);

always_comb

case(immsrc)

// I-type

2'b00:   immext = {{20{instr[31]}}, instr[31:20]};

// S-type (stores)

2'b01:   immext = {{20{instr[31]}}, instr[31:25], instr[11:7]};

// B-type (branches)

2'b10:   immext = {{20{instr[31]}}, instr[7], instr[30:25], instr[11:8], 1'b0};

// J-type (jal)

2'b11:   immext = {{12{instr[31]}}, instr[19:12], instr[20], instr[30:21], 1'b0};

default: immext = 32'bx; // undefined

endcase

endmodule

module flopr #(parameter WIDTH = 8)

(input  logic             clk, reset,

input  logic [WIDTH-1:0] d,

output logic [WIDTH-1:0] q);

always_ff @(posedge clk, posedge reset)

if (reset) q <= 0;

else       q <= d;

endmodule

module flopenr #(parameter WIDTH = 8)

(input  logic             clk, reset, en,

input  logic [WIDTH-1:0] d,

output logic [WIDTH-1:0] q);

always_ff @(posedge clk, posedge reset)

if (reset)   q <= 0;

else if (en) q <= d;

endmodule

module mux2 #(parameter WIDTH = 8)

(input  logic [WIDTH-1:0] d0, d1,

input  logic             s,

output logic [WIDTH-1:0] y);

assign y = s ? d1 : d0;

endmodule

module mux3 #(parameter WIDTH = 8)

(input  logic [WIDTH-1:0] d0, d1, d2,

input  logic [1:0]       s,

output logic [WIDTH-1:0] y);

assign y = s[1] ? d2 : (s[0] ? d1 : d0);

endmodule

module mux4 #(parameter WIDTH = 8)

(input  logic [WIDTH-1:0] d0, d1, d2, d3,

input  logic [1:0]       s,

output logic [WIDTH-1:0] y);

assign y = s[1] ? (s[0] ? d3: d2) : (s[0] ? d1 : d0);

endmodule

module mem(input  logic        clk, we,

input  logic [31:0] a, wd,

output logic [31:0] rd);

logic [31:0] RAM[63:0];

initial

$readmemh("riscvtest.txt",RAM);

assign rd = RAM[a[31:2]]; // word aligned

always_ff @(posedge clk)

if (we) RAM[a[31:2]] <= wd;

endmodule

module alu(input  logic [31:0] a, b,

input  logic [2:0]  alucontrol,

output logic [31:0] result,

output logic        zero);

logic [31:0] condinvb, sum;

logic        v;              // overflow

logic        isAddSub;       // true when is add or subtract operation

assign condinvb = alucontrol[0] ? ~b : b;

assign sum = a + condinvb + alucontrol[0];

assign isAddSub = ~alucontrol[2] & ~alucontrol[1] |

~alucontrol[1] &  alucontrol[0];

always_comb

case (alucontrol)

3'b000:  result = sum;         // add

3'b001:  result = sum;         // subtract

3'b010:  result = a & b;       // and

3'b011:  result = a | b;       // or

3'b100:  result = a ^ b;       // xor

3'b101:  result = sum[31] ^ v; // slt

3'b110:  result = a << b[4:0]; // sll

3'b111:  result = a >> b[4:0]; // srl

default: result = 32'bx;

endcase

assign zero = (result == 32'b0);

assign v = ~(alucontrol[0] ^ a[31] ^ b[31]) & (a[31] ^ sum[31]) & isAddSub;

endmodule

- Table 1 showing key signals for at least the first three instructions.

Table 1: Expected Operation (after two cycles of reset)

| Step | PC | Instr | State | Result | Result Notes |
| --- | --- | --- | --- | --- | --- |
| 3 | 00 | 00500113 | S0: Fetch | 4 | PC+4 |
| 4 | 04 | "" | S1: Decode | X | OldPC+Immediate |
| 5 | 04 | "" | S8: ExecuteI | X | ALUResult = x0 (0) + 5 = 5 |
| 6 | 04 | "" | S7: ALUWB | 5 | x2 = 5 |
| 7 | 04 | "" | S0: Fetch | 8 | PC+4 |
| 8 | 08 | 00c00193 | S1: Decode | X | OldPC+Immediate |
| 9 | 08 | "" | S8: ExecuteI | X | ALUResult = x0 (0) + 12 = 12 |
| 10 | 08 | "" | S7: ALUWB | 12 | x3 = 12 |
| 11 | 08 | "" | S0: Fetch | 0C | PC+4 |
| 12 | 0C | FF718393 | S1: Decode | X | OldPC+Immediate |
| 13 | 0C | "" | S8: ExecuteI | X | ALUResult = x3 (12) - 9 = 3 |
| 14 | 0C | "" | S7: ALUWB | -9 | x7 = 3 |
| 15 |  |  |  |  |  |
| 16 |  |  |  |  |  |
| 17 |  |  |  |  |  |
| 18 |  |  |  |  |  |
| 19 |  |  |  |  |  |
| 20 |  |  |  |  |  |
| 21 |  |  |  |  |  |
| 22 |  |  |  |  |  |
| 23 |  |  |  |  |  |
| 24 |  |  |  |  |  |
| 25 |  |  |  |  |  |
| 26 |  |  |  |  |  |
| 27 |  |  |  |  |  |
| 28 |  |  |  |  |  |
| 29 |  |  |  |  |  |
| 30 |  |  |  |  |  |
| 31 |  |  |  |  |  |
| 32 |  |  |  |  |  |
| 33 |  |  |  |  |  |
| 34 |  |  |  |  |  |
| 35 |  |  |  |  |  |
| 36 |  |  |  |  |  |
| 37 |  |  |  |  |  |
| 38 |  |  |  |  |  |
| 39 |  |  |  |  |  |
| 40 |  |  |  |  |  |
| 41 |  |  |  |  |  |
| 42 |  |  |  |  |  |
| 43 |  |  |  |  |  |

(Only fill in the notes column if it is helpful to you to understand what is happening and why.)

- Simulation waveforms (in the order listed in the lab) at least for the specified signals. Does your system pass your testbench? Circle or highlight the waves showing that the correct value is written to the correct address, and make sure it is legible.